Technical Information Notice No. 48

Category:		Information & Instruction

Application: 		To Masters of ALL vessels visiting California ports

Subject:	California At-Berth Emission Reduction Regulations

Visit Reporting Requirements for vessels

Dear Captain,

A) As of January 1, 2023, all container, reefer and passenger vessels visiting ports in California are required to comply with the “At-Berth Emission Reduction Regulations”.

Implementation Schedule:

• 	Container, reefer and passenger vessels: 2023

• 	Roll-on/roll-off vessels (auto carriers): 2025

- Tank vessels: 2025 (Los Angeles and Long Beach) and 2027 (Northern California)

Vessels subject to the requirements must use while at berth in any California-regulated port:

- CARB-Approved Emissions Control Strategy (CAECS), including shore power; or

- A Vessel Incident Event (VIE) for vessels unable to connect due to equipment failure; or

- Pay into the remediation fund to comply with the regulations.

Required Actions for compliance with inward reporting requirements set force in the at-berth regulations applicable to the container, reefer and passenger vessels:

- Once the vessel receives from CSM (POCR) – Control Room the notification informing about a forthcoming visit to a Californian Port, the Master of the vessel shall fill in the vessel’s specific information in the Terminal Operator Visit Report (Attachment California 13.5).

- At least seven (7) calendar days before arrival, the Master of the vessel shall communicate in writing (copying the local Charterer’s Agent, the responsible Technical Superintendent, Technical Fleet Manager and ECM) with the terminal operator and operator of the CAECS to coordinate the use of a CAECS, including the following:

• Request use of a CAECS (where applicable); and

• Supply the terminal operator and the operator of the CAECS with information about the compatibility of the vessel with the intended CAECS via the Terminal Operator Visit Report template, as completed under above action 1.

- Where the voyage is shorter than 7 days, implement the above actions 1 to 3 immediately upon receiving the notification from CSM (POCR) – Control Room.

Compliance with the At-Berth Emission Reduction Regulations is mandatory, for the time being, only for container, reefer and passenger vessels visiting Californian ports!

B) As of January 1, 2023 all vessels visiting Californian ports, are subject to visit reporting requirements to California Air Resources Board (CARB):

- Not later than 30 days after the vessel departs from the Californian port;

- A Vessel Operator Visit Report is required for each visit of the vessel to California;

- Vessels calling only at anchorage are not required to submit the “Vessel Operator Visit Report”.

Required Actions for compliance with outward Visit Reporting Requirements outlined in the at-berth regulations applicable to ALL vessels:

- Once the vessel receives from CSM (POCR) – Control Room the notification informing about the requirement to submit the Vessel Operator Visit Report after departure from the California Port, the Master of every vessel shall fill in the Vessel Operator Visit Report (Attachment California 13.6) and have it ready for the submission to CARB.

- The Master of every vessel shall submit shortly after departure  the completed Vessel Operator Visit Report (Attachment California 13.6) to CARB (Email:  shorepower@arb.ca.gov) with copy to the local Charterer’s Agent, responsible Technical Superintendent, Technical Fleet Manager and the respective ECM;

- Upon departure, the CSM (POCR) – Control Room will send a reminding notification to the vessel about the requirement to submit the Vessel Operator Visit Report.

Compliance with the “Visit Reporting Requirements” to California Air Resources Board (CARB) is mandatory for ALL vessels departing Californian ports!

Once the Master of every vessel, receives voyage orders to proceed to California, he shall liaise with the local Charterer’s Agents, the responsible Technical Superintendent, the Technical Fleet Manager and the ECM to ensure that all actions for compliance with the above A) and B) reporting requirements are fulfilled.

The Master shall ensure that all crewmembers onboard a vessel calling Californian ports are aware of the applicable requirements prior to arrival in California.

For the sake of compliance with the California At-Berth Emission Reduction Regulations and Visit Reporting Requirements, refer to the Port and Country Rules and Regulations, California Notice 13.

- Attachment California 13.5: Terminal Operator Visit Report;

- Attachment California 13.6: Vessel Operator Visit Report.

Columbia Shipmanagement